Certificate of Model Rocket Engines

Manufacturer: MRK

Germany

| Nr | Engine type | Eng. Class | Peak /N/ max. | Thrust time /s/ | Load mass /g/ | Prop. mass /g/ | Dia/ lenght /mm/ Price /EUR/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | 1/2A1-4 | 1/2A | 3 | 1.4 | 3.3 | 1.1 | 10.2 / 36 1.70 |
| 2. | 1/2A4-1 -0 | 1/2A | 9 | 0.5 | 3.2 | 0.9 | 10.2 / 36 " |
| 3. | A1-5 | A | 3 | 1.9 -2.1 | 4.5 | 2.0 | 10.2 / 36 " |
| 4. | A2-3 | A | 4 | 1.6 | 4.5 | 1.8 | 10.2 / 36 " |
| 5. | A2-4 | A | 4 | 1.6 | 4.5 | 1.8 | 10.2 / 36 " |
| 6. | A2-5 | A | 4 | 1.6 | 4.5 | 1.8 | 10.2 / 36 " |
| 7. | A3-1 -0 | A | 7 | 1.0 | 4.2 | 1.7 | 10.2 / 36 " |
| 8. | A4-1 -0 | A | 9 | 0.6 | 4.9 | 1.8 | 11.2 / 35 " |
| 9. | B2-5 | B | 5 | 2.6 | 6.9 | 3.8 | 10.2 / 55 2.33 |
| 10. | B4-1 -0 | B | 9 | 1.0 | 7.4 | 3.6 | 11.2 / 45 " |
| 11. | B5-2 | B | 10 | 1.0 | 8.1 | 3.6 | 13.3 / 45 " |
| 12. | C7-2 | C | 18 | 1.3 | 16.2 | 7.0 | 15.5 / 55 2.70 |
| 13. | 3/4D7-0 | D 15 Ns | 18 | 2.05 | 20.0 | 11.0 | 15.5 / 65 3.80 |
| 14. | D4-0 | D | 16 | 4.5 – 6.0 | 28.0 | 13.0 | 20.0 / 60 " |
| 15. | D11-3 | D | 18 | 1.9 | 30.0 | 14.0 | 20.0 / 60 " |
| 16. | 3/4E4-0RC | E 30 Ns | 16 | 5.3 | 36.0 | 18.0 | 20.0 / 90 4.70 |
| 17. | E4-0RC | E | 16 | 9+-0.5 | 41.0 | 24.0 | 20.0 / 90 6.00 |
| 18. | E20-2 | E | 30 | 2.2 | 62.0 | 28.2 | 26.0 / 95 7.00 |

The engines may be produced with other delay time too, including no delay time (0).

Each engine series is regular tested . Engines are tested already on the World and European Championships. Manufacturer guarantee quality of the products in the period of two years, under condition of proper storing and using.

All these engine types are in full conformity with the requirements of CIAM FAI Sporting code

vol. X , section 4d – Space Models.